# Introduction to Micro:bit

# Introduction

In this lesson, we will learn how use Micro:bit programming environment. We will create a simple led drawing and load it into the micro:bit controller.

# Necessarry:

- Micro:bit controller

- USB cable

- Micro:bit program or internet link in which to do programming

# Process!

- Follow the link:

Microsoft MakeCode for micro:bit (microbit.org)

- Choose a new project:

- A window will pop out in which you need to type the name of the project, then press the Create button:

- Next, a window opens in which you can already start programming. The program offers three types of programming: block programming, JavaScript and Python programming languages.

- Create any image and save the project.

  - From the basic choice, take the command on start

  - from the same basic choice take the command show leds.

  - LED lights can be lit by pressing on the corresponding LED square directly in the Show Leds command

- In turn, in the Python code language, it looks like this:

To depict a drawing, it is necessary to create an array that represents the entire LED block. Burning LED lamps are denoted by a hashtag, but non-combustible ones by a dot. In the first line, the command basic.show_leds is recorded, which means that the controller will show the burning lights all the time. Behind the command, open the parentheses and form an array in parentheses.

- When you press the "save" button, the project is automatically saved on the computer, but the program also offers the ability to connect the micro:bit controller with a USB wire to the computer and create a pair, thereby allowing you to immediately put the code in the controller.