# Movement Basics

# Introduction

In this lecture we will get acquainted with Kitronik :move motor extension for mocro:bit.

# Necessary:

- Micro:bit controller

- Kitronik :MOVE motor kit

- USB cable

- Micro:bit program or internet link in which to do programming

# Process!

- First, connect the parts of the :MOVE motor kit, creating a running robot and add to it micro:bit controller.

- In programm choose Extensions.

- Enter “kitronik” in the search engine and choose kitronik-move-motor

Now you have new blocks.

- Lets create the code to test robot movement with buttons. First we add a block on button A pressed from optional input and add move forvard at speed 0 from MOVE motor ...motors and specify driving speed 0 – 100.

- Choose block on button B pressed from input and add stop from MOVE motor ...motors.

You will get the following code:

! set a reasonable travel speed, because to stop the robot you need to press the B button on the controller, which means you will have to catch the robot first

- Download in micro:bit controller. Push button A on controller – robot start move. Press the B button to stop the robot

Python:

- Now try to rotate the robot by disassembling the same code, change the block move to the block spin.

- Download in micro:bit controller

Python:

- Now we try to go around thing.

- For that we need to create function. Go to Advanced – Functions – Make a function and write the function name and push Done.

- Take block set turn radius and set radius on standard from MOVE motor ...motors, insert it in function.

- Take block move right/left at speed 75 and insert it in function.

- Take block pause (ms) and set on 500ms and insert in function.

- Insert block stop in function.

- Create code, who will call the function. Take block on button A pressed from Input and from Functions insert call bigCircle

- Download in micro:bit controller and try

Python code:

# Challange:

- Choose a distance, start from speed 5 and measure how fast it can cover the selected distance with speed 5. Choose the step by which you will increase the speed and measure the same distance at different speeds, each time increasing the speed by the chosen step.

- Try turns by changing the radius and/or speed, find out what changed, find out how to get the exact desired turn.

- Insert a marker or pen that is the appropriate size Kitronik :move motor in the robot in a specially designated place and program the code to draw an S or other letter or drawing.

Python:

- Since the robot has no sensors to determine how many degrees the robot has turned, the only measure is time and speed. By changing these parameters, create code according to the following example, because of which the robot would go around an angular object with sides of equal length.

Python:

! The robot's accuracy can also be affected by battery charge, surface variations, slipperiness, debris on tires, motor wear and other factors, so accuracy may vary even from time to time.